3D U-Net Based Nasal Airway Segmentation from CT Volumes for Fluid Mechanics Analysis

Author: Asumboya Wilfred Ayine
Department: Biomedical Engineering
Institution: University of Ghana
Date: January 10, 2026

Abstract

Accurate segmentation of nasal airways from computed tomography (CT) scans is critical for understanding respiratory mechanics and planning clinical interventions. This project presents a deep learning pipeline for automated 3D nasal airway segmentation using a 3D U-Net architecture trained on volumetric CT data stored in NRRD format. The pipeline addresses the challenge of voxel-level classification in high-resolution 3D medical images, distinguishing airway tissue from background across 129 patient volumes. The model was trained on 103 volumes and validated on 26 volumes, achieving a Dice coefficient of 0.8855, IoU of 0.7982, and F1 score of 0.8855 at epoch 46. The segmented airway masks are intended for 3D printing and subsequent fluid mechanics studies to measure nasal breathing resistance. The pipeline incorporates comprehensive preprocessing, data augmentation, mixed precision training, learning rate scheduling, and early stopping mechanisms. Post-training analysis includes quantitative metrics, visual overlays, feature map analysis, error analysis of worst-performing cases, and 3D volumetric visualizations. This work demonstrates the effectiveness of deep learning for automated nasal airway segmentation and provides a foundation for computational fluid dynamics analysis of respiratory airflow.

Acknowledgements

……………………………………………………hehe, I will allow this for you to handle.

Table of Contents

- Introduction

- Literature Review

- Materials and Methods

- Results

- Discussion

- Conclusion and Future Work

- References

- Figures and Tables

1. Introduction

1.1 Background on Medical Image Segmentation

Medical image segmentation is the process of partitioning medical images into meaningful regions to facilitate clinical analysis, diagnosis, and treatment planning. In volumetric imaging modalities such as computed tomography (CT) and magnetic resonance imaging (MRI), segmentation involves classifying each three-dimensional pixel (voxel) into anatomical structures or pathological regions. Accurate segmentation is essential for quantitative analysis, surgical planning, radiation therapy, and computational modeling of physiological processes.

Traditional segmentation methods relied on manual delineation by radiologists or semi-automated techniques such as thresholding, region growing, and active contours. However, these approaches are time-consuming, subject to inter-observer variability, and often fail to capture complex anatomical boundaries in noisy or low-contrast images. The advent of deep learning has revolutionized medical image segmentation by enabling automated, accurate, and reproducible analysis of large-scale imaging datasets.

1.2 Importance of Volumetric (3D) Segmentation

Volumetric segmentation extends traditional 2D image segmentation into three dimensions, preserving spatial relationships and anatomical continuity across image slices. This is particularly important for structures such as airways, blood vessels, and organs that exhibit complex 3D geometry. Two-dimensional approaches that process individual slices independently often produce fragmented or inconsistent segmentations due to loss of inter-slice context.

Three-dimensional convolutional neural networks (CNNs) process entire volumetric inputs, capturing spatial dependencies in all directions. This allows models to learn hierarchical representations of anatomical features, improving segmentation accuracy and consistency. In the context of nasal airway segmentation, 3D models can accurately delineate intricate airway passages that vary significantly across axial, coronal, and sagittal planes.

1.3 Overview of Deep Learning Solutions

Deep learning, particularly convolutional neural networks, has achieved state-of-the-art performance in medical image segmentation tasks. The U-Net architecture, introduced by Ronneberger et al. in 2015, has become the dominant framework for biomedical image segmentation due to its encoder-decoder structure with skip connections that preserve both high-level semantic information and low-level spatial details. The 3D U-Net extends this architecture to volumetric data, using 3D convolutions to process depth, height, and width simultaneously.

Modern deep learning pipelines incorporate data augmentation to improve generalization, loss functions designed to handle class imbalance, and advanced training techniques such as mixed precision and learning rate scheduling. Frameworks like MONAI (Medical Open Network for AI) provide domain-specific tools for medical imaging, including preprocessing transformations, evaluation metrics, and pretrained models optimized for volumetric data.

1.4 Project Objectives and Scope

The primary objective of this project is to develop an automated deep learning pipeline for nasal airway segmentation from CT volumes. The segmented airway masks will be used to generate 3D printed physical models for fluid mechanics experiments aimed at measuring nasal breathing resistance. Accurate airway geometry is critical for computational fluid dynamics (CFD) simulations and experimental validation of airflow patterns, pressure distributions, and resistance characteristics in the nasal cavity.

Specific goals include:

- Loading and preprocessing 129 CT volumes stored in NRRD format with corresponding airway label masks

- Implementing a 3D U-Net architecture with residual units, batch normalization, and dropout regularization

- Training the model using a combined Dice and Binary Cross-Entropy loss function to handle class imbalance

- Evaluating performance using quantitative metrics: Dice coefficient, Intersection over Union (IoU), sensitivity, specificity, precision, and F1 score

- Conducting post-training analysis including error analysis, feature map visualization, and 3D volumetric views

This project demonstrates the application of deep learning to a specific biomedical engineering challenge, bridging medical imaging, computational modeling, and experimental biomechanics.

2. Literature Review

2.1 U-Net and 3D U-Net Architectures

The U-Net architecture was originally designed for biomedical image segmentation with limited training data. Its symmetric encoder-decoder structure uses downsampling to extract hierarchical features and upsampling to reconstruct spatial resolution, with skip connections that concatenate encoder features to decoder layers. This preserves fine-grained spatial information lost during downsampling, enabling precise localization of boundaries.

The 3D U-Net extends this concept to volumetric data by replacing 2D convolutions with 3D convolutions and 2D pooling with 3D pooling. Each encoder level reduces spatial dimensions while increasing feature channels, capturing progressively abstract representations. Decoder levels perform upsampling and concatenate skip connections from corresponding encoder levels. The final layer produces a voxel-wise classification map with the same spatial dimensions as the input volume.

Normalization techniques, like Batch Normalization, stabilize training primarily by smoothing the optimization landscape, which leads to more predictable and reliable gradients. Dropout regularization prevents overfitting by randomly deactivating neurons during training.

2.2 Common Loss Functions

Medical image segmentation often suffers from severe class imbalance, where foreground regions (e.g., airways) occupy a small fraction of the total volume compared to background. Standard loss functions like Binary Cross-Entropy (BCE) can lead to models biased toward the majority class.

The Dice loss, based on the Dice coefficient (a measure of overlap between prediction and ground truth), addresses class imbalance by focusing on the intersection and union of predicted and true positive regions. The Dice coefficient is defined as:

Dice = (2 × |Prediction ∩ Ground Truth|) / (|Prediction| + |Ground Truth|)

Dice loss is defined as 1 - Dice, making it differentiable for gradient-based optimization. However, Dice loss alone can produce unstable gradients for very small structures.

Combining Dice loss with Binary Cross-Entropy provides both overlap-based supervision and pixel-wise classification, balancing sensitivity to small structures with stable gradient propagation. This hybrid approach has been widely adopted in medical image segmentation.

2.3 Evaluation Metrics

Evaluation metrics quantify segmentation performance by comparing predicted masks to ground truth annotations:

- Dice Coefficient (Dice Score): Measures overlap between prediction and ground truth, ranging from 0 (no overlap) to 1 (perfect overlap).

- Intersection over Union (IoU / Jaccard Index): Ratio of intersection to union of predicted and true regions. IoU is more sensitive to false positives and false negatives than Dice.

- Sensitivity (Recall / True Positive Rate): Proportion of true airway voxels correctly identified. High sensitivity indicates the model captures most of the airway.

- Specificity (True Negative Rate): Proportion of true background voxels correctly identified. High specificity indicates the model does not over-segment.

- Precision (Positive Predictive Value): Proportion of predicted airway voxels that are correct. High precision indicates few false positives.

- F1 Score: Harmonic mean of precision and recall, balancing false positives and false negatives.

These metrics provide complementary perspectives on model performance, capturing different aspects of segmentation quality.

2.4 Visualization Techniques

Interpretability and error analysis are critical for understanding model behavior and identifying failure modes. Visualization techniques include:

- Prediction Overlays: Superimposing predicted masks on input images or ground truth masks to assess spatial accuracy.

- Feature Maps: Visualizing intermediate activations in encoder or decoder layers reveals learned features at different abstraction levels.

- Error Maps: Color-coded visualizations distinguishing true positives, false positives, and false negatives help identify systematic errors.

- 3D Volume Views: Orthogonal slices (axial, coronal, sagittal) provide multi-perspective assessment of volumetric predictions.

2.5 Existing Work on Volumetric Segmentation

Volumetric segmentation has been applied to diverse anatomical structures including brain tumors, liver lesions, cardiac chambers, and airways. Airway segmentation specifically has been addressed using region growing and deep learning methods. Challenges include handling varying airway caliber, bifurcations, partial volume effects, and imaging artifacts.

Recent deep learning approaches combine 3D CNNs with attention mechanisms, multi-scale feature fusion, and uncertainty quantification. Ensemble methods and test-time augmentation further improve robustness. Transfer learning from pretrained models on large datasets has shown promise for tasks with limited annotated data.

3. Materials and Methods

3.1 Dataset Description

3.1.1 Source and Type of Data

The dataset consists of 130 volumetric CT scans of the nasal cavity acquired from a hospital. Each CT volume captures the complete nasal airway anatomy in three dimensions. One volume was isolated for independent testing, leaving 129 volumes for training and validation. Each volume is stored in NRRD (Nearly Raw Raster Data) format, a standard medical imaging format that preserves voxel spacing, orientation, and metadata.

Corresponding to each CT volume is a manually annotated label volume where airway voxels are marked with positive values and background voxels are marked as zero. These ground truth annotations serve as supervision signals during training.

3.1.2 Volume Dimensions and Channels

CT volumes exhibit varying spatial dimensions across patients due to differences in scan protocols, field of view, and patient anatomy. Typical dimensions range from approximately 512×512×100 to 512×512×300 voxels, though exact dimensions vary per patient. Each volume contains a single intensity channel representing Hounsfield Units (HU), a standardized scale for CT attenuation.

3.1.3 Split: Training vs Validation

The 129 volumes were split into training and validation sets using an 80/20 stratified random split:

- Training Set: 103 volumes (80%)

- Validation Set: 26 volumes (20%)

The split ensures the validation set represents an independent sample for unbiased performance evaluation. No data augmentation is applied to the validation set to preserve the integrity of evaluation metrics.

3.1.4 Preprocessing Requirements

Preprocessing is essential to standardize heterogeneous CT volumes for neural network input. Key requirements include:

- Orientation Standardization: Aligning volumes to a consistent anatomical orientation (RAS: Right-Anterior-Superior).

- Voxel Spacing Normalization: Resampling to uniform spacing (1mm³) to ensure spatial consistency across patients.

- Intensity Normalization: Mapping Hounsfield Units to a normalized range [0, 1] to stabilize training.

- Spatial Resizing: Cropping or padding to a fixed spatial size compatible with GPU memory and network architecture.

- Binary Label Conversion: Converting multi-value label masks to binary (0 = background, 1 = airway).

3.2 Preprocessing

3.2.1 Loading NRRD Volumes

A custom MONAI transform (LoadNrrd) was implemented to read NRRD files using the pynrrd library. This ensures proper handling of volumetric metadata including voxel spacing, orientation, and coordinate systems. Volumes are converted to float32 precision and wrapped in MONAI MetaTensor objects to preserve metadata throughout the preprocessing pipeline.

3.2.2 Orientation and Spacing Standardization

The Orientationd transform reorients all volumes to RAS (Right-Anterior-Superior) coordinate system, ensuring consistent anatomical alignment. The Spacingd transform resamples voxels to uniform 1mm³ isotropic spacing using bilinear interpolation for images and nearest-neighbor interpolation for labels (to preserve discrete label values).

3.2.3 Intensity Normalization

CT intensities in Hounsfield Units typically range from -1000 (air) to +3000 (bone). For nasal airway segmentation, the relevant tissue range is approximately -1000 to +500 HU. The ScaleIntensityRanged transform clips values to this range and linearly maps them to [0, 1]:

Normalized Intensity = (HU - (-1000)) / (500 - (-1000))

Values outside the range are clipped to 0 or 1. This normalization improves gradient stability and convergence during training.

3.2.4 Cropping and Padding

To reduce computational cost and focus on relevant anatomy:

- CropForegroundd removes empty background regions by cropping around non-zero voxels in the image.

- CenterSpatialCropd extracts a centered region of interest (ROI) with fixed dimensions.

- SpatialPadd pads volumes smaller than the target size to ensure uniform input dimensions.

The final spatial size for all volumes is (192, 240, 64) voxels (depth × height × width), chosen to balance GPU memory constraints and anatomical coverage.

3.2.5 Data Augmentation (Training Only)

To improve model generalization and robustness to anatomical variations, the following augmentations are applied randomly during training:

| Transform | Parameters | Purpose |
| --- | --- | --- |
| RandRotated | range_x=0.15, range_y=0.15, range_z=0.15, prob=0.5 | Random 3D rotations to simulate different patient orientations |
| RandFlipd | spatial_axis=[0,1,2], prob=0.10 | Random flips along each axis to augment spatial variability |
| RandZoomd | min_zoom=0.9, max_zoom=1.1, prob=0.5 | Random zoom to simulate scale variations |

Augmentations use bilinear (Trilinear to be specific but bilinear is what is registered in MONAI for all forms of linear transformartions) interpolation for images and nearest-neighbor for labels to preserve label integrity.

3.2.6 Example Preprocessing Code

train_transforms = Compose([

LoadImaged(keys=["image", "label"]),

EnsureChannelFirstd(keys=["image", "label"]),

Orientationd(keys=["image", "label"], axcodes="RAS"),

Spacingd(keys=["image", "label"], pixdim=(1.0, 1.0, 1.0),

mode=("bilinear", "nearest")),

ScaleIntensityRanged(keys=["image"], a_min=-1000, a_max=500,

b_min=0.0, b_max=1.0, clip=True),

CropForegroundd(keys=["image", "label"], source_key="image"),

CenterSpatialCropd(keys=["image", "label"], roi_size=(192, 240, 64)),

SpatialPadd(keys=["image", "label"], spatial_size=(192, 240, 64)),

Lambdad(keys=["label"], func=lambda x: (x > 0).astype(np.float32)),

RandRotated(keys=["image", "label"], range_x=0.15, range_y=0.15,

range_z=0.15, prob=0.5, mode=("bilinear", "nearest")),

RandFlipd(keys=["image", "label"], spatial_axis=[0, 1, 2], prob=0.10),

RandZoomd(keys=["image", "label"], min_zoom=0.9, max_zoom=1.1,

prob=0.5, mode=("bilinear", "nearest")),

EnsureTyped(keys=["image", "label"], dtype=torch.float32),

])

3.3 Model Architecture

3.3.1 3D U-Net Overview

The 3D U-Net is an encoder-decoder architecture designed for volumetric segmentation. The encoder progressively downsamples the input volume while increasing feature channels, capturing hierarchical spatial representations. The decoder upsamples feature maps back to the original resolution while incorporating skip connections from the encoder to preserve fine-grained spatial details.

3.3.2 Encoder Path

The encoder consists of four downsampling levels, each containing:

- Two residual convolutional blocks with 3×3×3 kernels

- Batch normalization for training stability

- ReLU activation functions

- Strided convolutions (stride 2) for downsampling

Feature channels increase at each level: 16 → 32 → 64 → 128 → 256.

3.3.3 Decoder Path

The decoder mirrors the encoder with four upsampling levels, each containing:

- Transposed convolutions (stride 2) for upsampling

- Concatenation of skip connections from corresponding encoder levels

- Two residual convolutional blocks

- Batch normalization and ReLU activation

3.3.4 Residual Units

Residual blocks include shortcut connections that add the input directly to the output, enabling gradient flow through very deep networks. Each residual unit applies two 3×3×3 convolutions with batch normalization and ReLU, then adds the input via a skip connection.

3.3.5 Output Layer

The final layer is a 1×1×1 convolution that maps the decoder features to a single output channel representing the probability of each voxel belonging to the airway class. A sigmoid activation is applied during inference to produce probabilities in the range [0, 1].

3.3.6 Model Configuration Table

| Parameter | Value | Description |
| --- | --- | --- |
| Spatial Dimensions | 3 | 3D volumetric input |
| Input Channels | 1 | Single-channel CT intensity |
| Output Channels | 1 | Binary segmentation (airway vs background) |
| Encoder Channels | (16, 32, 64, 128, 256) | Feature maps at each encoder level |
| Decoder Channels | (128, 64, 32, 16) | Feature maps at each decoder level |
| Strides | (2, 2, 2, 2) | Downsampling factor at each level |
| Residual Units | 2 | Number of residual blocks per level |
| Dropout Rate | 0.1 | Dropout probability for regularization |
| Normalization | Batch Normalization | Stabilizes training |
| Activation | ReLU | Non-linear activation function |
| Total Parameters | 4,807,968 | Trainable model parameters |

3.3.7 Architecture Diagram

[Placeholder for 3D U-Net architecture diagram showing encoder path, decoder path, skip connections, and residual blocks]

3.4 Training

3.4.1 Loss Function

The model is trained using a combined Dice + Binary Cross-Entropy (BCE) loss function:

Loss = Dice Loss + BCE Loss

- Dice Loss: Optimizes overlap between prediction and ground truth, addressing class imbalance by focusing on the foreground (airway) region. Defined as 1 - Dice Coefficient.

- BCE Loss: Provides pixel-wise supervision, ensuring smooth gradients and stable convergence.

The combined loss leverages the strengths of both functions: Dice handles class imbalance while BCE provides fine-grained voxel-level feedback.

class DiceBCELoss(torch.nn.Module):

def __init__(self):

super().__init__()

self.dice = DiceLoss(sigmoid=True, squared_pred=True)

self.bce = BCEWithLogitsLoss()

def forward(self, pred, target):

return self.dice(pred, target) + self.bce(pred, target)

3.4.2 Optimizer

The Adam optimizer was selected for its adaptive learning rate and efficient convergence properties. Adam combines momentum and adaptive learning rates for each parameter, making it well-suited for high-dimensional optimization problems.

Optimizer: Adam
Learning Rate: 1e-4 (initial)
Weight Decay: 1e-5 (L2 regularization)

3.4.3 Learning Rate Scheduler

A ReduceLROnPlateau scheduler dynamically adjusts the learning rate based on validation performance. When the Dice score plateaus (no improvement for 5 consecutive validation checks), the learning rate is reduced by a factor of 0.5. This allows the model to fine-tune weights as training progresses.

Scheduler: ReduceLROnPlateau
Mode: max (maximize Dice score)
Factor: 0.5
Patience: 5 epochs
Minimum Learning Rate: 1e-6

3.4.4 Early Stopping

Early stopping prevents overfitting by monitoring validation Dice score. If no improvement occurs for 15 consecutive validation checks, training terminates and the best-performing model checkpoint is retained.

Early Stopping Patience: 15 epochs

3.4.5 Checkpointing

The model checkpoint with the highest validation Dice score is saved, including:

- Model state dictionary (weights and biases)

- Optimizer state dictionary

- Learning rate scheduler state

- Current epoch and best metric value

This ensures the final model represents the best generalization performance rather than the last training epoch.

3.4.6 Mixed Precision Training

Automatic Mixed Precision (AMP) reduces memory usage and accelerates training by using 16-bit floating-point operations where precision is not critical, while maintaining 32-bit precision for critical operations. PyTorch's torch.cuda.amp.GradScaler manages gradient scaling to prevent underflow in 16-bit arithmetic.

3.4.7 Hyperparameters Table

| Hyperparameter | Value | Description |
| --- | --- | --- |
| Maximum Epochs | 100 | Training stops at epoch 100 or early stopping |
| Batch Size | 1 | Number of volumes per training batch |
| Spatial Size | (192, 240, 64) | Fixed input volume dimensions (D×H×W) |
| Validation Interval | 2 | Validate every 2 epochs |
| Initial Learning Rate | 1e-4 | Starting learning rate for Adam |
| Final Learning Rate | 2.50e-5 | Learning rate at best epoch (epoch 46) |
| Weight Decay | 1e-5 | L2 regularization coefficient |
| Early Stop Patience | 15 | Epochs without improvement before stopping |
| Scheduler Patience | 5 | Epochs without improvement before LR reduction |
| Scheduler Factor | 0.5 | LR reduction multiplier |
| Dropout Rate | 0.1 | Probability of dropping neurons during training |
| Loss Function | Dice + BCE | Combined loss for class imbalance handling |

3.5 Post-Training Evaluation

3.5.1 Quantitative Metrics

Model performance is evaluated on the validation set using the following metrics:

| Metric | Formula | Description |
| --- | --- | --- |
| Dice Coefficient | 2×TP / (2×TP + FP + FN) | Overlap between prediction and ground truth |
| IoU (Jaccard Index) | TP / (TP + FP + FN) | Intersection over union ratio |
| Sensitivity (Recall) | TP / (TP + FN) | True positive rate (airway detection rate) |
| Specificity | TN / (TN + FP) | True negative rate (background detection rate) |
| Precision | TP / (TP + FP) | Positive predictive value |
| F1 Score | 2×(Precision×Recall) / (Precision + Recall) | Harmonic mean of precision and recall |
| Volume Similarity | 1 - | V_pred - V_gt |

Where TP = True Positives, FP = False Positives, FN = False Negatives, TN = True Negatives.

3.5.2 Visualization Techniques

Prediction Overlays:
Input CT slices, ground truth masks, predicted masks, and overlays (combining ground truth and prediction) are visualized side-by-side for qualitative assessment.

Feature Map Visualization:
Intermediate activations from encoder layers are extracted and visualized to understand learned features at different abstraction levels. Middle slices of 3D feature maps are displayed in grid layouts.

Error Analysis:
The 10 worst-performing validation samples (ranked by IoU) are analyzed in detail. Error maps distinguish:

- True Positives (TP): Green (correctly predicted airway)

- False Positives (FP): Red (incorrectly predicted as airway)

- False Negatives (FN): Yellow (missed airway regions)

Bar charts summarize error statistics per sample.

3D Volume Views:
Orthogonal slices (axial, coronal, sagittal) are extracted from 3D volumes to provide multi-perspective visualization of input, ground truth, and predictions.

4. Results

4.1 Training Performance

The model was trained for a total of 76 epochs before early stopping was triggered. The best model performance was achieved at epoch 46, with the following training characteristics:

- Initial Learning Rate: 1e-4

- Final Learning Rate (at epoch 46): 2.50e-5 (reduced by scheduler)

- Training Duration: 76 epochs (early stopping after 15 epochs without improvement)

4.1.1 Loss Curves

[Placeholder for Figure 1: Training and Validation Loss Curves]

Training loss decreased consistently from epoch 1 to approximately epoch 46, indicating effective learning. Validation loss followed a similar trend but stabilized around epoch 46, consistent with the best Dice score achieved at this epoch. The learning rate scheduler reduced the learning rate multiple times during training in response to plateaus in validation performance, allowing the model to fine-tune weights.

4.1.2 Metric Progression

[Placeholder for Figure 2: Training and Validation Dice Score Across Epochs]

Validation Dice score improved steadily during the first 46 epochs, reaching a peak of 0.8855 at epoch 46. After this point, no further improvement was observed for 15 consecutive validation intervals, triggering early stopping at epoch 76.

4.2 Validation Results

The best model (epoch 46) achieved the following performance on the 26-sample validation set:

| Metric | Value | Interpretation |
| --- | --- | --- |
| Dice Score | 0.8855 | High overlap between prediction and ground truth |
| IoU | 0.7982 | Strong intersection over union (79.82% overlap) |
| Sensitivity | 0.8906 | Model captures 89.06% of true airway voxels |
| Specificity | 0.9973 | Model correctly identifies 99.73% of background voxels |
| Precision | 0.8817 | 88.17% of predicted airway voxels are correct |
| F1 Score | 0.8855 | Balanced precision-recall performance |
| Volume Similarity | 0.9773 | Predicted and true airway volumes are highly similar |

These results demonstrate strong segmentation performance across all metrics. The high specificity (99.73%) indicates minimal false positive predictions in background regions. The sensitivity of 89.06% shows that the model successfully identifies the vast majority of airway voxels. The Dice score of 0.8855 is considered excellent for medical image segmentation tasks.

4.2.1 Sample Predictions

[Placeholder for Figure 3: Sample Validation Predictions - Overlay Visualizations]

Five representative validation samples show:

- Column 1: Input CT slice

- Column 2: Ground truth airway mask

- Column 3: Predicted airway mask

- Column 4: Overlay (ground truth in red, prediction in green, overlap in yellow)

The overlay visualizations reveal that predictions closely match ground truth annotations, with most errors occurring in small airway branches or regions with low contrast.

4.3 Feature Map Visualizations

[Placeholder for Figure 4: Feature Maps from Encoder Layers]

Feature maps extracted from encoder layers reveal the hierarchical feature learning process:

- Early layers (16 channels): Capture low-level features such as edges, boundaries, and intensity gradients

- Middle layers (64-128 channels): Detect anatomical patterns, airway shapes, and contextual structures

- Deep layers (256 channels): Encode high-level semantic representations of airway vs background regions

Visualization of middle slices from each encoder level shows progressive abstraction, with early layers responding to local intensity changes and deeper layers responding to global anatomical context.

4.4 Error Analysis

4.4.1 Error Statistics

Error analysis on the 10 worst-performing validation samples (ranked by IoU) reveals:

| Statistic | Value | Description |
| --- | --- | --- |
| Worst IoU | 0.4832 | Lowest IoU among validation samples |
| Best IoU | 0.8578 | Highest IoU among validation samples |
| Mean IoU | 0.7982 | Average IoU across validation set |
| Median IoU | 0.8210 | Median IoU (50th percentile) |
| Std IoU | 0.0759 | Standard deviation of IoU scores |

The worst-case IoU of 0.4832 indicates occasional failures where the model achieves only moderate overlap. However, the high median IoU (0.8210) and low standard deviation (0.0759) suggest that most predictions are accurate and consistent.

4.4.2 Error Visualizations

[Placeholder for Figure 5: Error Analysis - 10 Worst-Performing Samples]

Error maps for the 10 worst cases show:

- True Positives (Green): Correctly predicted airway voxels

- False Positives (Red): Background voxels incorrectly predicted as airway

- False Negatives (Yellow): Airway voxels incorrectly predicted as background

Common failure modes include:

- Under-segmentation in narrow airway branches: Small peripheral airways are sometimes missed, particularly in regions with low CT contrast or partial volume effects

- Over-segmentation near boundaries: The model occasionally extends predictions slightly beyond true airway boundaries

- Inconsistent segmentation in noisy regions: Imaging artifacts or motion blur can cause localized prediction errors

These failure cases provide insights for future improvements, such as incorporating multi-scale attention mechanisms or refining annotations in challenging regions.

4.5 3D Volume Views

[Placeholder for Figure 6: 3D Volumetric Visualizations - Orthogonal Views]

Three orthogonal views (axial, coronal, sagittal) are displayed for representative validation samples:

- Axial View (XY plane): Horizontal cross-section showing nasal passages from superior to inferior

- Coronal View (XZ plane): Frontal cross-section showing nasal cavity depth

- Sagittal View (YZ plane): Side cross-section showing left-right nasal symmetry

Each view displays input CT slice, ground truth mask, predicted mask, and overlay. The 3D visualizations confirm that the model maintains anatomical consistency across all three spatial dimensions, producing coherent 3D airway structures rather than fragmented 2D predictions.

5. Discussion

5.1 Analysis of Performance Metrics

The achieved Dice score of 0.8855 and IoU of 0.7982 demonstrate excellent segmentation performance for nasal airway extraction from CT volumes. These metrics are comparable to or exceed reported performance in the literature for similar anatomical structures. The high sensitivity (89.06%) indicates that the model successfully captures the majority of true airway voxels, which is critical for downstream applications such as 3D printing and fluid dynamics analysis where complete airway geometry is essential.

The exceptionally high specificity (99.73%) reflects the class imbalance in the dataset: background voxels vastly outnumber airway voxels, making it easier for the model to correctly classify background regions. While high specificity is desirable, the balance between sensitivity and specificity (reflected in the F1 score of 0.8855) is more informative for assessing overall segmentation quality.

The volume similarity metric of 0.9773 indicates that the predicted airway volume is very close to the true airway volume, which is particularly important for applications requiring accurate volumetric measurements. This suggests the model is not systematically over-segmenting or under-segmenting the airway.

5.2 Insights from Feature Map Analysis

Visualization of feature maps from encoder layers provides interpretability into the model's learned representations. Early encoder layers respond to local intensity gradients, edges, and boundaries, consistent with the low-level features expected from convolutional filters. Middle layers capture intermediate anatomical patterns such as tubular structures and bifurcations characteristic of airways. Deep encoder layers encode high-level semantic concepts, distinguishing airway regions from surrounding tissues based on global context.

The progressive abstraction from low-level to high-level features mirrors the hierarchical organization of visual perception in biological systems and validates the U-Net's design philosophy. Skip connections between encoder and decoder layers allow the decoder to access both high-level semantic information (from deep encoder layers) and fine-grained spatial details (from early encoder layers), enabling precise boundary localization.

5.3 Common Errors and Failure Cases

Error analysis of the 10 worst-performing validation samples reveals systematic failure modes:

1. Under-segmentation in narrow peripheral airways:
Small airway branches with diameters approaching the voxel resolution are sometimes missed or incompletely segmented. This is likely due to partial volume effects, where voxels contain mixtures of airway and tissue, producing ambiguous intensity values. Future work could address this by incorporating multi-scale or multi-resolution architectures that process both local and global context.

2. Over-segmentation near airway boundaries:
The model occasionally extends predictions slightly beyond true airway boundaries, particularly in regions with smooth intensity transitions. This may result from the combined Dice + BCE loss function, which prioritizes overlap (Dice) over strict boundary adherence. Post-processing with morphological operations (e.g., erosion) could refine boundaries.

3. Errors in low-contrast or noisy regions:
Imaging artifacts, motion blur, or low signal-to-noise ratio can introduce prediction inconsistencies. Data augmentation with simulated noise during training may improve robustness to these artifacts.

5.4 Comparisons to Previous Work

The Dice score of 0.8855 achieved in this project is competitive with state-of-the-art airway segmentation methods reported in the literature. Studies using similar 3D U-Net architectures for airway or vascular segmentation typically report Dice scores in the range of 0.80-0.90, depending on dataset characteristics, annotation quality, and evaluation protocols.

The use of a combined Dice + BCE loss function is consistent with best practices in medical image segmentation, addressing the class imbalance problem inherent in airway segmentation tasks. The incorporation of residual units, batch normalization, and dropout further aligns with modern deep learning practices for improving generalization and training stability.

5.5 Recommendations for Improvement

Several strategies could further improve segmentation performance:

1. Multi-class segmentation:
Extending the model to distinguish between different airway regions (e.g., nasal vestibule, turbinates, nasopharynx) would provide richer anatomical information and enable region-specific analysis.

2. Larger and more diverse datasets:
Training on additional patient volumes, including cases with anatomical variations, pathologies, or different imaging protocols, would improve generalization to unseen data.

3. Attention mechanisms:
Incorporating attention modules (e.g., squeeze-and-excitation blocks or spatial attention) could help the model focus on relevant anatomical features while suppressing noise and artifacts.

4. Ensemble methods:
Combining predictions from multiple models trained with different initializations, augmentations, or architectures could improve robustness and reduce variance.

5. Post-processing refinement:
Morphological operations, conditional random fields (CRFs), or graph-based refinement could smooth boundaries and enforce topological constraints (e.g., airway connectivity).

6. Transfer learning:
Fine-tuning a model pretrained on large medical imaging datasets (e.g., Medical Segmentation Decathlon) could accelerate convergence and improve performance on limited data.

6. Conclusion and Future Work

6.1 Summary of Findings

This project successfully developed an automated deep learning pipeline for 3D nasal airway segmentation from CT volumes. The 3D U-Net model achieved a Dice coefficient of 0.8855, IoU of 0.7982, and F1 score of 0.8855 on a validation set of 26 patient volumes, demonstrating excellent segmentation performance. The model maintains high sensitivity (89.06%) for airway detection and exceptional specificity (99.73%) for background classification, producing anatomically consistent 3D airway structures suitable for downstream applications.

The pipeline incorporates comprehensive preprocessing (orientation standardization, voxel spacing normalization, intensity normalization, and spatial resizing), robust training strategies (combined Dice + BCE loss, Adam optimizer, learning rate scheduling, early stopping, and mixed precision training), and extensive post-training analysis (quantitative metrics, feature map visualization, error analysis, and 3D volume views).

6.2 Significance of Results

Accurate nasal airway segmentation has significant implications for biomedical engineering and clinical applications:

- 3D Printing and Fluid Mechanics: The segmented airway masks enable creation of patient-specific 3D printed models for experimental fluid mechanics studies. These physical models facilitate measurement of nasal breathing resistance, pressure-flow relationships, and validation of computational fluid dynamics (CFD) simulations.

- Clinical Diagnosis and Surgical Planning: Automated airway segmentation supports quantitative assessment of nasal obstruction, surgical planning for functional rhinoplasty, and evaluation of post-operative outcomes.

- Computational Modeling: High-quality segmentation provides accurate geometric boundaries for CFD simulations of nasal airflow, heat transfer, and particle deposition, which are critical for understanding respiratory physiology and drug delivery.

6.3 Potential Improvements and Future Work

Future work could extend this project in several directions:

1. Multi-class nasal airway segmentation:
Expanding the model to segment individual nasal structures (turbinates, septum, sinuses) would provide richer anatomical detail and enable region-specific airflow analysis.

2. Integration of larger datasets:
Acquiring and annotating additional CT volumes, particularly from diverse patient populations and imaging protocols, would improve model generalization and clinical applicability.

3. Real-time deployment and clinical workflow integration:
Optimizing the model for inference speed (e.g., through model compression, quantization, or efficient architectures) would enable real-time segmentation during clinical workflows.

4. Uncertainty quantification:
Incorporating Bayesian deep learning or ensemble methods to estimate prediction uncertainty would provide clinicians with confidence intervals and highlight ambiguous regions requiring expert review.

5. Longitudinal analysis:
Extending the pipeline to analyze longitudinal CT scans (e.g., pre- and post-surgical comparisons) would enable quantitative assessment of treatment efficacy.

7. Automated quality control:
Developing automated methods to detect and flag low-quality segmentations or annotation errors would improve reliability in clinical settings.

In conclusion, this project demonstrates the feasibility and effectiveness of deep learning for automated nasal airway segmentation from 3D CT volumes. The high segmentation accuracy, comprehensive evaluation framework, and potential for clinical translation establish a strong foundation for future research in medical image analysis and respiratory biomechanics.

8. References

- Ronneberger O, Fischer P, Brox T. U-Net: Convolutional Networks for Biomedical Image Segmentation. In: Medical Image Computing and Computer-Assisted Intervention (MICCAI). 2015. p. 234-241.

- Çiçek Ö, Abdulkadir A, Lienkamp SS, Brox T, Ronneberger O. 3D U-Net: Learning Dense Volumetric Segmentation from Sparse Annotation. In: Medical Image Computing and Computer-Assisted Intervention (MICCAI). 2016. p. 424-432.

- Sudre CH, Li W, Vercauteren T, Ourselin S, Jorge Cardoso M. Generalised Dice Overlap as a Deep Learning Loss Function for Highly Unbalanced Segmentations. In: Deep Learning in Medical Image Analysis and Multimodal Learning for Clinical Decision Support. 2017. p. 240-248.

- Milletari F, Navab N, Ahmadi SA. V-Net: Fully Convolutional Neural Networks for Volumetric Medical Image Segmentation. In: 2016 Fourth International Conference on 3D Vision (3DV). 2016. p. 565-571.

- Isensee F, Jaeger PF, Kohl SAA, Petersen J, Maier-Hein KH. nnU-Net: a self-configuring method for deep learning-based biomedical image segmentation. Nature Methods. 2021;18(2):203-211.

- Cardoso MJ, Li W, Brown R, et al. MONAI: An open-source framework for deep learning in healthcare. arXiv preprint arXiv:2211.02701. 2022.

- Kingma DP, Ba J. Adam: A Method for Stochastic Optimization. In: International Conference on Learning Representations (ICLR). 2015.

- He K, Zhang X, Ren S, Sun J. Deep Residual Learning for Image Recognition. In: IEEE Conference on Computer Vision and Pattern Recognition (CVPR). 2016. p. 770-778.

- Ioffe S, Szegedy C. Batch Normalization: Accelerating Deep Network Training by Reducing Internal Covariate Shift. In: International Conference on Machine Learning (ICML). 2015. p. 448-456.

- Selvaraju RR, Cogswell M, Das A, Vedantam R, Parikh D, Batra D. Grad-CAM: Visual Explanations from Deep Networks via Gradient-Based Localization. In: IEEE International Conference on Computer Vision (ICCV). 2017. p. 618-626.

9. Figures and Tables

List of Figures

- Figure 1: Training and Validation Loss Curves

- Figure 2: Training and Validation Dice Score Across Epochs

- Figure 3: Sample Validation Predictions - Overlay Visualizations (5 samples)

- Figure 4: Feature Maps from Encoder Layers

- Figure 5: Error Analysis - 10 Worst-Performing Samples with Error Maps

- Figure 6: 3D Volumetric Visualizations - Orthogonal Views (axial, coronal, sagittal)

- Figure 7: 3D U-Net Architecture Diagram (encoder-decoder with skip connections)

List of Tables

- Table 1: Hyperparameters Used in Training

- Table 2: Model Architecture Configuration

- Table 3: Quantitative Evaluation Metrics on Validation Set

- Table 4: Error Analysis Statistics

- Table 5: Layer Configuration (channels, kernel size, activation)